vc/30-Jul-26

### Week 41 starting 06 Oct 03

### SM18 Operation SCHEDULE Meeting (Tuesdays)

Schedule for this week :

| ### BENCH | ### CURRENT # MAGNET | ### STATUS ### Today(07/10//03) | Forward look, & Next Magnet |
| --- | --- | --- | --- |
| ### TBA1 | ### 3047 | Should be removed | # Bench to be de-activated – preparation for D bench |
| ### TBA2 | ### 3042 | Should be removed | ### Next Magnet 1020 |
| ### TBC1 | ### 3045 | On Standby | Start Cold Tests on Fri & during w’end when SSS not used |
| ### TBC2 | ### SSS1 | cold | Cold tests |
| ### TBE1 | ### 2002 | Warm HV Tests | ### Disconnect Wed. & prepare for New without Anticryostats-3044? |
| ### TBE2 | ### 3039 | cold | ### Cold Tests till Thurs? |
| ### TBF1 | ### 2025 after F2 done? |  | ### Prepare For ### Commissioning AFTER F2 |
| ### TBF2 | ### 2025 | Cold | ### Commissioning TESTS |

Test Coordinator this week:  Marco+Nikolai;  MTM  Tech. on call:  (16-3602) : Maryline

- Situation From Cryogenics:  J-P Lamboy or ACR Rep.

- Other General Issues

  - New RRR Doc.;

  - F Bench Commissioning & Red folder

  - “  “”           “”                 ( console from CO ?).,

  - Interactions in Traveller & Info

## New Test Sequence Update Web form & Display(GHH)

  - AOB

- Problems from Operation Last week (Stephane + Test Supervisor )

| Date | Problem | Type | Application/soft. | Action/advise |
| --- | --- | --- | --- | --- |
| Tuesday 30_M | TBA2 : No data from quench (MIITS), |  | AQA | ? |
| Tuesday 30_A | TBE1:IAP @warm failed.: D20 connector not connected | MTM |  | Connected and OK |
| Tuesday 30_A | TBA2: No read current during special LHC cycle between 760 A and 3 kA | MTM | Display | Read current in black during the ramp for LHC cycles. |
| Wednesday 1_A | TBE2: IAP @warm forgotten. Config file not ready. | MTM |  | Prep 7 has not to be performed before prep6 . Wait for the config file to perform prep 6. |
| Wednesday 1_A | Cryogenic problem (warmer not working) | MTM |  | Magnets lost the cryo OK and go to 4.4 K |
| Wednesday 1_N | MM (TBA2) : 10 kA-LHC cycle launched in Qloc position. Over ranges observed. | MTM |  | Test aborted and repeated. |
| Wednesday 1_N Thursday 2_M | MM: After export data for MM, display is hung | AB/CO | Display | Pedro informed. Investigation on progress |
| Thursday 2_M | TBA1: Unable to launch “thermal cycle “ from TEMA | AB/CO |  | One INCAA card was removed and MXI crate powered off. Solved by Magnus. |
| Thursday 2_N Friday_3_M | TBA1: HV test at 1.9 K failed: problem with humidity in current leads | MTM |  | Test MB/QH and QH/ground. OK Test coordinator gave oK to continue the test at cold. But before warm up complete HV test (current leads disconnected) performed. |
| Thursday 2_N | TBA1: IAP at cold . MXI error | MTM |  | HF data acquisition card missing |
| Thursday 2_N | TBA2 : Quench at 6.5 kA to warm up LF triggered but no File not created | MTM/AB/CO? | LF |  |
| ?Thursday 2_N | Cryogenic : cut of the power in the Liquefactor. Then Temperature not stable. | MTM | Liquefactor | NO HV test possible, |
| Friday 3_A | TBE1: IAP @ cold. Problem to launch the test, | MTM | IAP | Switch off and on the power supply. |
| Friday 3_N | TBA1: Problem in Power Converter. | SL/PO |  | Loops were changed in RMA,RME, RMC without informing us. Loop of SSS. Giorgio changed the loop. |
| Saturday 4_M | TBE1: HV test at cold failed But data could not be found | AB/CO? |  | Test repeated. |
| Saturday 4_M | TBE1: HV test at cold failed Ext Passive bus bar. | MTM |  | Giorgio and Stephane informed. OK to continue the test. Non conformity was opened. |
| Saturday 4_M | TBE1:D_2_U-L potaim card did not work | MTM |  | Giorgio informed. Replaced by one in the cluster F |
| Saturday 4_A | TBA2:IAP@warm failed MXI communication error for HF acqusition | MTM-AB/CO? | IAP | Acquisition no getting armed HF and LF acquisition rebooted. |
| Sunday 5_M | TBA1:RRR measurement DVM DAQ system blocked | AB/CO | DVM DAQ | DVM DAQ rebooted. |
| Sunday 5_N | Electrical power cutStop of the liquefactor . Cryo not OK for all benches | Liquefactor |  |  |