Vc/31-Jul-26

### Week 19 starting 03 May 04

### SM18 Operation Meeting (Tuesdays)

Bench status  and tests for now & next few days :

| Bench | Name | Monday 03/05 Afternoon status |  |
| --- | --- | --- | --- |
| TBB1 | 3083 | New Arrival | with Anti C |
| TBB2 | _ | New ? |  |
| TBC1 | MBBL3089 | PT 10 Min. NRJ 11850 A |  |
| TBC2 | SSS015 | EX1 Extra Test |  |
| TBD1 | MBBL2049 | ICS 3 Traveller after warm test |  |
| TBD2 | _ | New ? |  |
| TBE1 | MBBL3123 | PT 5.2 1.5 KA Quench HF |  |
| TBE2 | MBBL1110 | PT 12 Warm Up |  |
| TBF1 | MBBL1043 | ICS 3 Traveller after warm test | with Anti C |
| TBF2 | _ | New ? |  |

Test Coordinator this week: J Miles ; MTM support on call  Guy:  (16-3602).

4 Dipoles  Disconnected after tests last week    ( Sat Midnight to Sat midnight)

- Situation From Cryogenics:  J-P Lamboy or ACR Rep.

- B2 mech. work for legs completed on Monday ? ( J Halik)

- Preparation of D Cluster for SSS  ( J.Halik?)

- General Problems last week :

SSS work :

Nair + Sridhar , SSW etc etc..

| Date | Bench | Problem | Type | Application/soft | Action/advise |
| --- | --- | --- | --- | --- | --- |
| Tuesday 27_N | TBC1 | During IAP permanent external trigger on potaim card. | ICS |  | Ewald informed. One taps not connected. ICS informed. The team opened the magnet and found that the external lead connection was not OK. Solved |
| Tuesday 27_A | ALL | No water in SM18 |  |  | TCR called. OK after 1 hour. |
| Tuesday 27_A | TBC2 | Communication between PLC and power converter not working on SSS. | AB/CO? AB/PO? |  | Ewald informed. Recalibration of the power supply asked by E.Garde and authorised by Nicolay? |
| Tuesday 27_A | TBC2 | Discrepancy between T read in TEMA and in through cryogeny. | MAS |  | Thermometer calibration of the SSS not present in MTF? |
| Tuesday 27_A | TBF2, TBE2 | Magnets 3104 and 3110: TT821 sensor coefficient not present in MTF. | MAS |  | Ewald informed Stéphane. He contacted directly P.Lienard. Problem solved for the future. |
| Wednesday 28_M | TBC2 | SSS powered. Ramp to 5 kA but SPA. PLC reading reports “cryo loss”. | ACR |  | Pressure on the lines going down due to restart of cold compressor tests in BA7? |
| Thursday 29_A | TBB2 | QH control crate not scan because sticker missing. | MTM |  | Equipment entered manually by Ewald. |
| Friday 30_A | TBD1 | 3072: Quench at 4.2 K to warm up. Faulty quench. Flat signals in HF off: D1 L-U D1-D2… | MTM | HF | Stéphane checked the quench. Apparently consistency in the direct signals picture. He advised Ewald to check the HF recording |
| Friday 30_A | TBD2 | SPA occurred at 8629 A. | MTM? ACR? ABPO? |  |  |
| Friday 30_A Sunday 2_M | TBB1 | Switch from to CC position possible. PLC error during IAP Rotary switch command box is not working. (no light, no answer when you press the buttons) | MTM |  | Ewald called. He managed to solve punctually the problem. Guy will inform Giorgio Monday for an action. |
| Saturday 1_N | TBF1 | IAP at cold: HF and LF armed. Only LF received the triggers. HF acquisition problem | AB/CO | HF | Reset the crates and restart TEMA. Not OK Ewald called. He changed one card and reset the MXI. |
| Saturday_1_M Sunday 2_M |  | Video TV display not working. |  |  | Clear instructions are needed. Call Dirk Bakker? |
| Sunday 2_N | TBB1 | STMS not updated because SM18 Database is not working | ? |  |  |